Title: Predicting Gravity Anomalies in Topographically Complex Terrain: Machine Learning and Least Squares Collocation Modeling in Central Macedonia, Greece

Abstract:

The adoption of Artificial Intelligence (AI) and Machine Learning (ML) methodologies is fundamentally reshaping geodetic research. Recognizing this transformative potential, the Global Geodetic Observing System (GGOS) recently established a dedicated Focus Area on Aligning with the goal of Artificial Intelligence for Geodesy (AI4G) of GGOS to develop higher-resolution and more accurate geodetic products through the assimilation of complex, multi-source datasets, this study addresses a fundamental challenge: the precise determination of the gravity field and the local geoid in regions exhibiting extreme topographical and morphological variations and heterogenous subsurface structure.

The Region of Central Macedonia (RCM) in northern Greece serves as a highly demanding study area for this purpose. Spanning from extensive coastal zones and the Chalkidiki peninsula to some of the country's highest elevations including Mount Olympus, Mount Voras, and Mount Athos, the region's topographic characteristics complicate gravimetric modeling. Furthermore, achieving high-resolution gravity mapping in such demanding terrains is frequently hindered by a severe lack of dense terrestrial data. Alternative acquisition methods, such as airborne gravimetry, remain prohibitively expensive, while the direct downward continuation of satellite gravity data to the Earth's surface continues to be a mathematically complex subject under ongoing investigation.

To address these limitations, this study develops and compares three gravity prediction strategies over the RCM. First, a conventional geodetic solution is produced using Least Squares Collocation (LSC) modeling, serving as the reference baseline. Second, a pure ML strategy is implemented, in which gravity anomalies are predicted directly from spatial, topographic, and geological predictors. Third, a hybrid geodetic–ML strategy is introduced, where XGBoost regression is used to model local residual gravity anomalies after the removal of an independent long-wavelength reference gravity signal. In this hybrid approach, the removed component is subsequently restored, allowing the final solution to combine geodetic prior information with the ability of ML to model non-linear local variations. The modeling leverages a dataset of 16,117 irregularly distributed land gravity observations. To capture the complex physical realities of the region, the spatial coordinates and orthometric heights of these observations are integrated with high-resolution geological and lithological information provided by the Hellenic Survey of Geology and Mineral Exploration.

To ensure an unbiased assessment, all three modeling strategies are evaluated using a common validation framework. A subset of the terrestrial gravity observations is withheld from model training and used exclusively for independent accuracy assessment. First, the predictive accuracy of the ML models is assessed directly against this reserved, independent subset of the irregularly distributed ground-truth observations. Second, to evaluate spatial continuity and interpolation performance, both the ML models and LSC are utilized to predict gravity anomalies on a regular grid covering the entire study area. A comprehensive comparative analysis of the resulting grids is then conducted to demonstrate how modern AI approaches perform against established geodetic baselines, providing a robust framework for continuous gravity field modeling in topographically diverse environments.