Title: Predicting Gravity Anomalies in Topographically Complex Terrain: Machine Learning and Least Squares Collocation Modeling in Central Macedonia, Greece

Abstract:

The adoption of Artificial Intelligence (AI) and Machine Learning (ML) is reshaping geodetic research. Recognizing this transformative potential, the Global Geodetic Observing System (GGOS) aims to develop higher-resolution and more accurate geodetic products through the assimilation of complex, multi-source datasets. This study addresses a fundamental challenge in that respect, i.e., the precise determination of the gravity field and the local geoid in regions exhibiting extreme topographical and morphological variations.

The Region of Central Macedonia (RCM) in northern Greece serves as a highly demanding study area for this purpose. Spanning from extensive coastal zones and the Chalkidiki peninsula to some of the country's highest elevations, the region's topography complicate gravity field modeling. Furthermore, achieving high-resolution gravity mapping in such terrains is frequently hindered by a severe lack of dense terrestrial data. Alternative acquisition methods, such as airborne gravimetry, remain prohibitively expensive, while the direct downward continuation of satellite gravity data (i.e.GOCE), hinder the inhered attenuation of the gravity field signal due to the satellite altitude, hence provide only a band-limited representation to the medium wavelengths (~up to 80 km).

To address these limitations, this study develops and compares three gravity prediction strategies.. First, a conventional geodetic solution is produced using Least Squares Collocation (LSC) modeling, serving as the reference baseline. Second, a pure ML method is implemented, in which gravity anomalies are predicted directly from spatial, topographic, and geological predictors. Third, a hybrid geodetic–ML strategy, where XGBoost regression is used to model local residual gravity anomalies after the removal of an independent long-wavelength reference gravity signal.. The modeling uses a dataset of 16,117 irregularly distributed land gravity observations. To capture the complex physical realities of the region, the spatial coordinates and orthometric heights of these observations are integrated with high-resolution geological and lithological information provided by the Hellenic Survey of Geology and Mineral Exploration.

To ensure an unbiased assessment, all three modeling strategies are evaluated using a common validation framework. A subset of the terrestrial gravity observations is withheld from model training and used exclusively for independent accuracy assessment. First, the predictive accuracy of the ML models is assessed directly against this reserved, independent subset. Second, to evaluate spatial continuity and interpolation performance, both the ML models and LSC are utilized to predict gravity anomalies on a regular grid covering the entire study area. A comprehensive comparative analysis of the resulting grids is then conducted to demonstrate if modern AI approaches perform worse against established geodetic baselines, in topographically diverse environments.